# Lab Exercise 6: AHB UART Peripheral

# Overview

In this lab, we design and implement an AHB UART peripheral to communicate with a PC or laptop. The work includes:

- Hardware design and implementation:

- Design and implement the UART hardware mechanism to interface with a PC or laptop.

- Design and implement the AHB bus interface for the peripheral.

- Prototype the hardware onto a FPGA.

- Software programming:

- Program the Cortex-M0 processor; access the UART peripheral using assembly language.

- Demonstrate the SoC:

- Send/receive information to/from a host (PC or laptop).

- Analyze the behavior of the peripheral using an on-chip hardware debugging tool.

# Details

## Hardware

### system components

The hardware components of the SoC include:

- An Arm Cortex-M0 microprocessor

- An AHB-Lite system bus

- Three AHB peripherals

  - A BRAM module

  - A VGA peripheral

  - A UART peripheral to interface with a host

SoC Peripherals

### file structure

The files that will be used for the peripheral are listed below:

FILES USED BY VGA PERIPHERAL

| File name | Description |
| --- | --- |
| AHBUART.v | The top module of the UART peripheral, includes the AHB interface |
| baudgen.v | Generate system ticks for a fixed transmission baud rate, e.g., 19200 bps |
| uart_rx.v | UART receiver; receives eight sequential bits and translates them to 8-bit parallel data |
| uart_tx.v | UART transmitter; sends 8-bit data in sequential bits |
| fifo.v | A FIFO to buffer the data to be sent and data that has been received |

AHBUART.v

baudgen.v

uart_rx.v

uart_tx.v

fifo.v

fifo.v

### block diagram

UART Peripheral Block Diagram

- UART transmitter

  - Reads data (in byte) from the transmitter FIFO

  - Converts a single byte data to sequential bits

  - Sends bits to the tx pin, clocked in a fixed rate provided from the baud generator

- UART receiver

  - Receives the sequential bits from the rx pin using the clock generated from the baud generator

  - Reassembles the bits to a single byte

  - Writes the received byte to the receiver FIFO

- UART FIFO

  - Temporally buffers the data to be sent or the data that has been received

### memory map

The default memory map of the peripherals is listed below:

MEMORY MAP OF PERIPHERALS

| Peripheral | Base address | End address | Size |
| --- | --- | --- | --- |
| BRAM | 0x0000_0000 | 0x00FF_FFFF | 16MB |
| VGA | 0x5000_0000 | 0x50FF_FFFF | 16MB |
| UART | 0x5100_0000 | 0x51FF_FFFF | 16MB |

PERIPHERAL REGISTERS

| Register | Base address | Size |
| --- | --- | --- |
| Data | 0x5100_0000 | 4 bytes |
| FIFO status | 0x5100_0004 | 4 bytes |

- Data register

Used for both input and output data

- FIFO status register

  - Bit0: Rx FIFO empty

If empty, the processor cannot read from the FIFO.

  - Bit1: Tx FIFO full

If full, the processor must wait before writing to the FIFO.

## Software

The main code should be written in assembly and should perform the following:

- Initialize the interrupt vector.

- Display a string (e.g., “TEST”) at the console region (same as previous lab).

- Plot four pixels at the four corners of the image region (same as previous lab).

Then, it should repeat the following:

- Wait until a character is received (the receive FIFO is not empty).

- Print the received text to both UART and VGA.

An example of the demo:

Demo Example

# HARDWARE debugging

## on-chip debugging

Use the on-chip debugging tool to sample and analyze the signals at run-time. Suggested signals are as follows.

Towards AHB bus:

- HADDR[31:0]

- HWDATA[31:0]

- HRDATA[31:0]

- HWRITE

- HREADY

- HSIZE[2:0]

- HTRANS[1:0]

- HRESP

Towards the UART:

- FIFO_tx_empty

- FIFO_rx_empty

- UART_tx

- UART_rx

- Tx_data[7:0]

- Rx_data[7:0]

# extension work

Here are some extra things that you can do:

- Add configuration registers to the UART peripheral, whereby the processor can configure the peripheral by modifying its configuration registers; for example,

  - Change the UART baud rate.

- Add parity bit for the UART transmission.

- Send/receive files to/from a host using UART.